Minor Project: 8-Bit ALU Design

Final Documentation

1. Project Overview

The aim of this project was to design and verify an 8-bit Arithmetic Logic Unit (ALU) using Verilog HDL. The ALU accepts two 8-bit inputs, A and B, and a 3-bit SEL input to select one of eight operations. The design also provides carry_out and zero_flag outputs.

2. Operations Implemented

| SEL | Operation | Function |
| --- | --- | --- |
| 000 | Addition | A + B |
| 001 | Subtraction | A - B |
| 010 | AND | A AND B |
| 011 | OR | A OR B |
| 100 | XOR | A XOR B |
| 101 | NOT | NOT A |
| 110 | Left Shift | A << 1 |
| 111 | Right Shift | A >> 1 |

3. Design Decisions

• The design was divided into separate Verilog modules for addition, subtraction, AND, OR, XOR, NOT and shifting.

• The top-level module alu_top connects the operation modules and selects the required result using the 3-bit SEL input.

• The ALU was implemented as combinational logic, so no clock or storage registers are required.

• The zero_flag becomes active when the ALU result is zero. The carry_out output is used for arithmetic carry information.

4. Testbench and Simulation

The testbench was written to test all eight ALU operations. Three test vectors were used for each operation, giving a total of 24 test vectors. The simulation was run in Questa and the outputs were checked using waveforms.

| Check | Input / Operation | Observed Result |
| --- | --- | --- |
| Addition | 10 + 5 | 15 |
| Addition carry | 255 + 1 | 00, carry_out = 1 |
| Subtraction | 10 - 5 | 5 |
| Zero flag | 5 - 5 | 00, zero_flag = 1 |
| AND | AA AND CC | 88 |
| OR | AA OR CC | EE |
| XOR | AA XOR CC | 66 |
| NOT | NOT AA | 55 |
| Left Shift | 05 << 1 | 0A |
| Right Shift | FF >> 1 | 7F |

The complete 24-test waveform was saved separately as project evidence. The waveform confirms that SEL values 000 through 111 select the required arithmetic, logical and shift operations.

5. Synthesis and Resource Utilization

The design was synthesized in Vivado for the xc7a12tcpg238-1 FPGA device. The synthesis result shows that the ALU uses only a small amount of FPGA logic.

| Resource | Used / Available | Utilization |
| --- | --- | --- |
| Slice LUTs | 40 / 8000 | 0.50% |
| F7 Muxes | 8 / 7300 | 0.11% |
| Slice Registers | 0 / 16000 | 0.00% |
| Block RAM | 0 / 20 | 0.00% |
| DSPs | 0 / 40 | 0.00% |

6. Timing Analysis

The synthesized combinational timing was checked in Vivado. Since the ALU is combinational, the timing check was taken from an input port to an output port.

| Timing Item | Result |
| --- | --- |
| Source | B[1] |
| Destination | zero_flag |
| Path Type | Max at Slow Process Corner |
| Data Path Delay | 8.464 ns |
| Logic Delay | 5.338 ns |
| Route Delay | 3.126 ns |
| Logic Levels | 8 |

7. Results and Observations

• All eight ALU operations produced the expected outputs during simulation.

• The updated testbench uses 24 test vectors, which gives three test vectors for each ALU operation.

• The carry_out output became active for the overflow addition test 255 + 1.

• The zero_flag became active when the ALU result was zero.

• The synthesized design used 40 Slice LUTs and 8 F7 muxes.

• No Slice Registers, Block RAM or DSP blocks were used because the current ALU is a combinational design.

• The maximum input-to-output propagation delay observed in the synthesized design was 8.464 ns.

8. Conclusion

The 8-bit ALU was designed, simulated and synthesized successfully. The design supports arithmetic, logical and shift operations using a 3-bit select input. Simulation results matched the expected values, and the Vivado synthesis report showed low FPGA resource usage. This project provided practical experience with Verilog modules, testbenches, waveform verification, RTL schematic analysis, synthesis, resource utilization and timing analysis.