8-Bit ALU - Resource Utilization

Vivado synthesis result

| Top module | alu_top |
| --- | --- |
| FPGA part | xc7a12tcpg238-1 |
| Tool | Vivado 2026.1 |

I checked the synthesis utilization report in Vivado and noted the main resources used by the ALU.

| Resource | Used | Available | Utilization | Note |
| --- | --- | --- | --- | --- |
| Slice LUTs | 40 | 8000 | 0.50% | Used for ALU logic |
| LUT as Logic | 40 | 8000 | 0.50% | All used LUTs are logic |
| LUT as Memory | 0 | 5000 | 0.00% | No LUT memory used |
| Slice Registers | 0 | 16000 | 0.00% | No registers used |
| F7 Muxes | 8 | 7300 | 0.11% | Used for selection logic |
| Block RAM Tile | 0 | 20 | 0.00% | No Block RAM used |
| RAMB36 / FIFO | 0 | 20 | 0.00% | Not used |
| RAMB18 | 0 | 40 | 0.00% | Not used |
| DSPs | 0 | 40 | 0.00% | No DSP blocks used |

Result: The 8-bit ALU uses 40 Slice LUTs and 8 F7 muxes. It does not use flip-flops, Block RAM, or DSP blocks. This is expected because the current ALU is a combinational design.

Timing Analysis

I checked the maximum input-to-output delay of the synthesized ALU in Vivado. Since this ALU is combinational, the timing check is based on the propagation delay from an input to an output.

| Timing Item | Result |
| --- | --- |
| Source | B[1] |
| Destination | zero_flag |
| Path Type | Max at Slow Process Corner |
| Data Path Delay | 8.464 ns |
| Logic Delay | 5.338 ns |
| Route Delay | 3.126 ns |
| Logic Levels | 8 |

Timing result: The maximum propagation delay observed in the synthesized design is 8.464 ns, from input B[1] to output zero_flag.