RISC Processor - Control Signal Truth Table

Control signals generated by control_unit.v

The control unit checks the 4-bit opcode and generates the signals needed for register write, memory access, ALU input selection and ALU operation.

| Instruction | Opcode | RegWrite | MemRead | MemWrite | ALUSrc | MemtoReg | ALUOp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ADD | 0000 | 1 | 0 | 0 | 0 | 0 | 00 |
| SUB | 0001 | 1 | 0 | 0 | 0 | 0 | 01 |
| AND | 0010 | 1 | 0 | 0 | 0 | 0 | 10 |
| OR | 0011 | 1 | 0 | 0 | 0 | 0 | 11 |
| LOAD | 0100 | 1 | 1 | 0 | 1 | 1 | 00 |
| STORE | 0101 | 0 | 0 | 1 | 1 | 0 | 00 |

Signal meaning

• RegWrite = 1: write the result into the destination register.

• MemRead = 1: read data from data memory.

• MemWrite = 1: write register data into data memory.

• ALUSrc = 0: ALU second input comes from a register; ALUSrc = 1: it comes from the immediate value.

• MemtoReg = 1: memory data is written back to the register file; 0 selects the ALU result.

• ALUOp selects ADD, SUB, AND or OR inside the RISC ALU.