Major Project: RISC Processor Design

Final Project Report

1. Project Overview

The aim of this project was to design and verify a simple single-cycle RISC processor using Verilog HDL. The processor uses a 16-bit instruction word and an 8-bit datapath. It includes a Program Counter, Instruction Memory, Control Unit, Register File, ALU and Data Memory. The processor was tested using a small instruction program containing arithmetic, logical and memory instructions.

2. Main Architecture

| Module | Purpose |
| --- | --- |
| Program Counter | Holds the current instruction address and increments every clock cycle. |
| Instruction Memory | Stores 16-bit instructions and outputs the instruction selected by the PC. |
| Control Unit | Decodes the opcode and generates control signals. |
| Register File | Contains eight 8-bit general-purpose registers, R0 to R7. |
| ALU | Performs ADD, SUB, AND and OR operations. |
| Data Memory | Provides storage for LOAD and STORE instructions. |

3. ISA and Design Decisions

A fixed 16-bit instruction format was selected before coding. A 4-bit opcode identifies the instruction, and 3-bit register fields allow eight registers. The design supports four R-type operations and two I-type memory operations.

| Instruction | Opcode | Type | Operation |
| --- | --- | --- | --- |
| ADD | 0000 | R-type | Rd = Rs1 + Rs2 |
| SUB | 0001 | R-type | Rd = Rs1 - Rs2 |
| AND | 0010 | R-type | Rd = Rs1 AND Rs2 |
| OR | 0011 | R-type | Rd = Rs1 OR Rs2 |
| LOAD | 0100 | I-type | Rd = Memory[Rs1 + imm] |
| STORE | 0101 | I-type | Memory[Rs1 + imm] = Rd |

• The processor was implemented as a single-cycle design to keep the datapath and control logic easy to understand and verify.

• LOAD and STORE use an immediate value for address calculation, while arithmetic and logical instructions use register operands.

• The STORE instruction required separate selection of the source register whose value is written to Data Memory.

• The testbench clock period was set to 10 ns.

4. Verilog Modules and Integration

| File | Function |
| --- | --- |
| risc_alu.v | ADD, SUB, AND and OR operations |
| register_file.v | 8 x 8-bit register file |
| instruction_memory.v | 256 x 16-bit instruction memory |
| program_counter.v | 8-bit program counter |
| control_unit.v | Opcode decoding and control generation |
| data_memory.v | 256 x 8-bit data memory |
| risc_top.v | Top-level integration of all modules |
| risc_tb.v | Testbench and instruction program |

5. Test Program and Simulation Results

The processor was simulated in Questa using a seven-instruction program. The program first loads two values from memory, performs arithmetic and logical operations, and finally stores the ADD result back to memory.

| PC | Instruction | Result |
| --- | --- | --- |
| 0 | LOAD R1, [R0 + 0] | R1 = 10 |
| 1 | LOAD R2, [R0 + 1] | R2 = 5 |
| 2 | ADD R3, R1, R2 | R3 = 15 |
| 3 | SUB R4, R1, R2 | R4 = 5 |
| 4 | AND R5, R1, R2 | R5 = 0 |
| 5 | OR R6, R1, R2 | R6 = 15 |
| 6 | STORE R3, [R0 + 2] | Memory[2] = 15 |

Simulation result: TEST PASSED

The waveform showed the PC progressing from 0 to 7, register values updating after write-back, MemRead active during LOAD instructions, MemWrite active during STORE, and Memory[2] changing to 15 after the STORE instruction.

6. Control Signals

| Inst. | RegWrite | MemRead | MemWrite | ALUSrc | MemtoReg | ALUOp |
| --- | --- | --- | --- | --- | --- | --- |
| ADD | 1 | 0 | 0 | 0 | 0 | 00 |
| SUB | 1 | 0 | 0 | 0 | 0 | 01 |
| AND | 1 | 0 | 0 | 0 | 0 | 10 |
| OR | 1 | 0 | 0 | 0 | 0 | 11 |
| LOAD | 1 | 1 | 0 | 1 | 1 | 00 |
| STORE | 0 | 0 | 1 | 1 | 0 | 00 |

7. Execution Cycle Count

The processor is a single-cycle design, so each instruction completes in one processor clock cycle. With a 10 ns clock period, the seven-instruction test program uses seven processor cycles, equal to 70 ns of clock time after reset is released.

| Instruction | Cycle Count |
| --- | --- |
| ADD | 1 |
| SUB | 1 |
| AND | 1 |
| OR | 1 |
| LOAD | 1 |
| STORE | 1 |

8. Timing Analysis

The synthesized design was checked in Vivado using a 10 ns clock constraint. The timing summary reported positive setup and hold slack, and Vivado confirmed that all specified timing constraints were met.

| Timing Item | Result |
| --- | --- |
| Clock period | 10.000 ns |
| WNS | 7.524 ns |
| TNS | 0.000 ns |
| WHS | 0.140 ns |
| THS | 0.000 ns |
| WPWS | 4.500 ns |
| TPWS | 0.000 ns |
| Timing status | All user specified timing constraints are met |

9. Challenges Faced

• Defining one 16-bit instruction format that supports both register operations and memory operations.

• Selecting the correct register source for the STORE instruction while keeping the register file interface simple.

• Verifying that register write-back and memory write operations happen on the correct clock edges.

• Checking PC progression, control signals, register values and memory values together during simulation.

10. Results and Observations

• All required instructions - ADD, SUB, AND, OR, LOAD and STORE - were executed successfully.

• The final simulation automatically reported TEST PASSED.

• The PC incremented correctly and the expected register and memory values were observed.

• The processor met the 10 ns timing constraint with positive setup and hold slack.

• The modular Verilog structure made the design easier to test and understand.

11. Future Scope

• Add a basic 2-stage Fetch/Execute pipeline as an enhancement.

• Add branch and jump instructions.

• Expand the instruction set and memory size.

• Implement the design on an FPGA board and test it with physical inputs and outputs.

12. Conclusion

The RISC processor was designed, integrated, simulated and synthesized successfully. The processor supports the required arithmetic, logical and memory instructions, and the test program produced the expected register and memory results. Timing analysis also confirmed that the design meets the selected 10 ns clock constraint. This project provided practical experience with ISA design, datapath integration, control logic, memory operations, testbench development, waveform verification and timing analysis.