Major Project: RISC Processor

Instruction Set Architecture (ISA) Document

1. ISA Overview

For this project, a simple 16-bit instruction format is used. The processor uses an 8-bit datapath and eight general-purpose registers. The instruction set includes the six operations required for the project: ADD, SUB, AND, OR, LOAD and STORE.

2. Register File Specification

| Item | Specification | Meaning |
| --- | --- | --- |
| Number of registers | 8 | R0 to R7 |
| Register width | 8 bits | Each register stores one 8-bit value |
| Register address width | 3 bits | Three bits select one of 8 registers |

3. Instruction Types

Two instruction types are used in this design:

• R-type: used for ADD, SUB, AND and OR. These instructions use two source registers and one destination register.

• I-type: used for LOAD and STORE. These instructions use a base register together with a 6-bit immediate offset.

4. 16-bit Instruction Format

R-type format

| [15:12] | [11:9] | [8:6] | [5:3] | [2:0] |
| --- | --- | --- | --- | --- |
| Opcode | Rd | Rs1 | Rs2 | 000 |

I-type format for LOAD / STORE

| [15:12] | [11:9] | [8:6] | [5:0] |
| --- | --- | --- | --- |
| Opcode | Rd / Src | Base Register | Immediate |

5. Opcode Table

| Instruction | Type | Opcode | Operation |
| --- | --- | --- | --- |
| ADD | R-type | 0000 | Rd = Rs1 + Rs2 |
| SUB | R-type | 0001 | Rd = Rs1 - Rs2 |
| AND | R-type | 0010 | Rd = Rs1 AND Rs2 |
| OR | R-type | 0011 | Rd = Rs1 OR Rs2 |
| LOAD | I-type | 0100 | Rd = Memory[Rs1 + imm] |
| STORE | I-type | 0101 | Memory[Rs1 + imm] = Rd |

6. Field Meaning

| Field | Bits | Purpose |
| --- | --- | --- |
| Opcode | [15:12] | Selects the instruction operation. |
| Rd | [11:9] | Destination register for R-type and LOAD. For STORE, this is the register whose value is written to memory. |
| Rs1 | [8:6] | First source register for R-type, and base register for LOAD/STORE. |
| Rs2 | [5:3] | Second source register for R-type instructions. |
| Immediate | [5:0] | 6-bit offset used by LOAD and STORE. |

7. Example Instructions

| Example | Meaning |
| --- | --- |
| ADD R1, R2, R3 | R1 = R2 + R3 |
| SUB R4, R4, R1 | R4 = R4 - R1 |
| LOAD R2, [R0 + 1] | Load memory at address R0 + 1 into R2 |
| STORE R2, [R0 + 2] | Store the value in R2 into memory at address R0 + 2 |

8. ISA Design Note

The ISA is defined before Verilog coding so that the opcode values, source register fields, destination register fields and memory instruction format remain fixed during processor implementation.